MP5 Mission 3: Lua Geofence Breach Alert

Implementation and Explanation

# Overview

The task is to write a Lua script that runs on the ArduPilot autopilot inside SITL and detects when the drone breaches its geofence. The script monitors the drone position every second and sends alerts to the ground control station when the drone gets close to the fence and when it actually crosses it.

The geofence has two parts in this implementation. The first part is a circular fence with a 50 meter radius around the home location. The second part is a maximum altitude of 30 meters. The script reads both limits at runtime, so if the operator changes the limits the script picks up the new values automatically.

# Setup

## Enable Lua scripting in SITL

ArduPilot has scripting support built in but it is disabled by default. Two parameters need to be set.

param set SCR_ENABLE 1
param set SCR_HEAP_SIZE 200000

SCR_ENABLE turns scripting on. SCR_HEAP_SIZE gives the Lua interpreter enough memory to run scripts. The values get stored in the SITL eeprom file so they survive across launches as long as the launch does not use the wipe flag.

After setting these, restart SITL once. From the next launch onwards, ArduPilot will look for .lua files in the scripts folder inside the directory where SITL was launched. For ArduCopter SITL the launch directory is normally ardupilot/ArduCopter, so the scripts folder is at ardupilot/ArduCopter/scripts.

## Configure the geofence

The fence is configured through ArduPilot parameters. None of the limits need to be drawn on a map. They are pure numbers.

param set FENCE_ENABLE 1
param set FENCE_TYPE 7
param set FENCE_RADIUS 50
param set FENCE_ALT_MAX 30
param set FENCE_ACTION 0

FENCE_ENABLE turns the fence subsystem on. FENCE_TYPE is a bitmask, where 1 is altitude max, 2 is circular radius, and 4 is polygon. The value 7 enables all three but only altitude max and radius are used in this demo because polygon would need a shape file uploaded. FENCE_RADIUS sets the circular fence to 50 meters. FENCE_ALT_MAX sets the altitude ceiling to 30 meters. FENCE_ACTION 0 means the autopilot does not take any automatic action like RTL or Land when the fence is breached, the script handles all the alerting itself.

## Place the script

The script file is named geofence_alert.lua and is placed in ardupilot/ArduCopter/scripts. After saving the file, the script is loaded by typing the following in MAVProxy.

scripting restart

MAVProxy then prints AP: Scripting: restarted followed by AP: GeoFence script loaded which confirms the script is running.

# How the Script Works

The script has four main parts. First it reads the fence parameters into Parameter objects so they can be queried efficiently every tick. Second it defines an update function that runs once per second. Third the update function reads the drone position and computes how far it is from home and how high it is. Fourth it checks the position against the fence limits and sends alerts when the drone is close to the limit or has crossed it.

## Reading parameters

local FENCE_ENABLE   = Parameter()
local FENCE_RADIUS   = Parameter()
local FENCE_ALT_MAX  = Parameter()
FENCE_ENABLE:init('FENCE_ENABLE')
FENCE_RADIUS:init('FENCE_RADIUS')
FENCE_ALT_MAX:init('FENCE_ALT_MAX')

Parameter() creates an empty parameter object. The init call binds it to a real ArduPilot parameter by name. After this the script can call get on each object to read the current value without doing a slow string lookup every time.

## Reading the drone position

local pos  = ahrs:get_position()
local home = ahrs:get_home()
if pos == nil or home == nil then
    return update, CHECK_INTERVAL_MS
end

ahrs:get_position returns the current drone location as a Location object. ahrs:get_home returns the home location. Both can return nil if the autopilot does not have a position fix yet, so the script checks for nil and skips the rest of the update if either is missing.

## Computing distance and altitude

local dist  = pos:get_distance(home)
local alt_m = (pos:alt() - home:alt()) / 100.0

Location:get_distance returns the horizontal distance between two Location objects in meters. Location:alt returns the altitude in centimeters in the location's frame. Subtracting home altitude gives altitude above home, and dividing by 100 converts from centimeters to meters.

## Breach detection logic

The script compares the current distance against the radius limit and the altitude against the altitude limit. There are three states for each limit.

- Below 80 percent of the limit. The drone is comfortably inside, no alerts are sent.

- Between 80 percent and 100 percent of the limit. A warning is sent the first time this state is entered.

- Above 100 percent of the limit. A breach alert is sent the first time this state is entered.

The state is tracked using boolean flags such as was_breached_radius and was_warning_radius. These flags prevent the script from sending the same message every second while the drone stays in the same state. When the drone returns to a safer state, the flags reset and a recovery message is sent.

## Severity levels

The gcs:send_text function takes a severity level as its first argument. Lower numbers mean higher importance.

- Severity 0 is EMERGENCY. Used for breach messages so they appear in red in MAVProxy and Mission Planner.

- Severity 4 is WARNING. Used when the drone is close to the limit.

- Severity 6 is INFO. Used for the script loaded message and recovery messages.

## Scheduling

The script schedules itself by returning two values from the update function. The first value is the function to run next, which is update itself. The second value is the delay in milliseconds before running again. This pattern lets the script run periodically without using a busy loop.

return update, CHECK_INTERVAL_MS  -- inside update, schedule next run
return update, 2000               -- at end of script, first run after 2s

# Demonstration

## What the video shows

The video demonstration covers two breach scenarios: an altitude breach and a radius breach. The MAVProxy console is visible the whole time so the alert messages can be read as they appear.

## Altitude breach

The drone is armed in GUIDED mode and commanded to take off to 35 meters. Since FENCE_ALT_MAX is 30 meters, the drone will breach the ceiling on the way up.

mode GUIDED
arm throttle
takeoff 35

As the drone climbs past 24 meters, which is 80 percent of the 30 meter ceiling, the script prints:

AP: GeoFence WARNING: approaching alt ceiling (24.4m of 30m)

As soon as the drone crosses 30 meters the breach message appears:

AP: *** FENCE BREACH (altitude): 31.6m above 30m ceiling ***

ArduPilot's built in fence system also prints its own messages such as AP: Max Alt fence breached. These come from the C++ side and confirm that the autopilot agrees with the script's view of the breach.

After bouncing back below 30 meters the script prints:

AP: GeoFence: back below alt ceiling

## Radius breach

With the drone hovering at a safe altitude, a velocity command is sent to fly the drone east at 5 meters per second.

velocity 0 5 0

As the drone moves east, the horizontal distance from home grows. After about 8 seconds the drone is around 40 meters out and the script prints:

AP: GeoFence WARNING: approaching radius (40.x of 50m)

After about 10 seconds the drone crosses the 50 meter radius and the breach message appears:

AP: *** FENCE BREACH (radius): 50.x out of 50m fence ***

The drone is then commanded back toward home with velocity 0 -5 0, and once it is back within 80 percent of the radius the script prints:

AP: GeoFence: back inside radius

# Files Submitted

- geofence_alert.lua, the script.

- Video demonstration showing both an altitude breach and a radius breach with the alert messages visible in MAVProxy.

- This document, explaining how the script works and how it was tested.